Table S1: Wearable devices for construction health and safety management

| Wearable devices | Explanation |
| --- | --- |
| Smartwatch | Smartwatches are computerized devices or small computers intended to be worn on the wrist, and have expanded functionality that is often related to communication. Smartwatches can help to monitor environmental conditions such as noise level and reporting unsafe exposure to workers (Mardonova and Choi, 2018; and Nnaji et al., 2021). |
| Smart Eyewear | Smart glasses or eyewear are used for various applications in optical head-mounted displays (OHMDs), heads-up displays (HUDs), Virtual Reality (VR), Augmented Reality (AR), Mixed Reality (MR), and smart contact lenses. The displays of smart glasses can be monocular if the information is displayed for a single eye or binocular if an image is displayed for both (Mardonova and Choi, 2018) |
| Fitness Tracker | Fitness trackers are typically worn on the wrist, chest, or ears, and are designed to monitor and track outdoor sport activities and measure fitness-related metrics, such as the speed and distance of running, exhalation, pulse rate, and sleeping habits (Cadmus-Bertram, 2017; Mardonova and Choi, 2018) |
| Smart Clothing (Sensor-equipped safety vest, Sensor-equipped safety helmet, Sensor incorporated into footwear) | Smart clothing includes a broad list of wearables, ranging from sportswear, chest straps, medical apparel, work wear monitoring apparel, military apparel, and e-textiles. Smart clothing is in the form of shirts, socks, yoga pants, shoes, bow ties with secret cameras, helmets, and caps with a wide range of sensors and features (Mardonova and Choi, 2018; and Nnaji et al., 2021). With regards to construction, these wearable sensing technologies are attached to PPEs for safety risk detection and health monitoring (Arabshahi et al, 2021). |
| Wearable Camera | Wearable cameras are well-suited for creating first-person videos and photos in real time. Two major types of wearable cameras are used: small cameras that can be attached to either the body or clothes, or can even be worn in the ear, and larger cameras with mounting attachments to affix to caps or helmets (Khoa, 2015). |
| Wearable Medical Device | A wearable medical device typically consists of one or more biosensors used to monitor a variety of physiological data to prevent disease, provide early diagnoses, and facilitate treatment and home rehabilitation. Digital healthcare wearable devices are often grouped together with other wearables, such as activity monitors, smartwatches, and smart clothing, and are all intended to help gather important data concerning the health of the patient using non-invasive sensors installed on the device (Patel et al., 2012; Mardonova and Choi, 2018; and Nnaji et al., 2021). |
| Dust sensor | Dust sensors can monitor fine particle concentration and protect workers against excessive respirable dust. The dust sensor could be fixed on the helmets and safety glasses and connected with smartphones and smartwatches (Smaoui, 2018; Arabshahi et al, 2022). |
| Rain sensor | As an environmental sensor, the rain sensor monitors the rain fall in a particular environment. It can be attached to the body for early warning against precipitation (Swan, 2012) |
| Smart bracelet/band | A smart bracelet/band is a typical health and safety device worn on the wrist for variety of activities including hazard detection, location detection and health monitoring of site operatives (Awolusi et. al , 2018). |

Table by authors

Table S2: Profile of respondents

| Category | Classification | Frequency | % |
| --- | --- | --- | --- |
| Gender | Male | 92 | 85 |
|  | Female | 16 | 15 |
| Total |  | 108 | 100 |
| Profession |  |  |  |
|  | Architect | 12 | 11.1 |
|  | Builder | 44 | 40.7 |
|  | Quantity Surveyor | 40 | 37.0 |
|  | Health and safety officers | 6 | 5.6 |
|  | Engineer | 2 | 1.9 |
|  | Others | 4 | 3.7 |
| Total |  | 108 | 100 |
| Employer type |  |  |  |
|  | Government Agency | 48 | 44 |
|  | Consultancy | 20 | 19 |
|  | Contracting | 40 | 37 |
| Total |  | 108 | 100 |
| Level of Education |  |  |  |
|  | Bsc/B.Tech | 44 | 41 |
|  | HND | 10 | 9 |
|  | MSc/M.Tech | 36 | 33 |
|  | PhD | 18 | 17 |
| Total |  | 108 | 100 |

Table by authors

Table S3: Awareness versus Adoption

| No. | Variables | Awareness | Adoption |
| --- | --- | --- | --- |
| 1 | Sensor-equipped safety vest | Somewhat aware | Rarely adopted |
| 2 | Smart eyewear | Somewhat aware | Rarely adopted |
| 3 | Sensor-equipped safety helmet | Somewhat aware | Rarely adopted |
| 4 | Smart watch | Aware | Rarely adopted |
| 5 | Smart bracelet/band | Somewhat aware | Rarely adopted |
| 6 | Fitness tracker | Somewhat aware | Rarely adopted |
| 7 | Wearable camera | Aware | Rarely adopted |
| 8 | Wearable medical device | Somewhat aware | Rarely adopted |
| 9 | Dust sensor | Slightly aware | Rarely adopted |
| 10 | Rain sensor | Slightly aware | Rarely adopted |
| 11 | Sensor incorporated into footwear | Slightly aware | Rarely adopted |

Table by authors