Supplementary Material 1. CLIMB2 questionnaire

The questionnaire is designed to facilitate self-assessment within an organization. It contains 107 questions divided into 8 areas. Below each answer is an empty box to mark the response that most closely reflects the truth regarding each question. In cases where a given practice does not apply to the organization for any reason, "N/A" should be entered in the middle field. Below is a glossary of selected terms to assist in completing the questionnaire:

Frontloading – Uneven distribution or allocation of resources (e.g., costs, effort, etc.), with greater allocation at the beginning of the product development process to avoid higher costs of changes in later phases of the process.

PDM/PLM – Systems supporting product data management.

KBE (Knowledge-Based Engineering) – Application of knowledge-based system technologies in the field of design and manufacturing. KBE is essentially engineering based on knowledge models. A knowledge model uses knowledge representation to represent design process artifacts instead of or in addition to conventional programming techniques and databases. In traditional CAD systems and industrial systems, each application often has its own, slightly different model. Having a standardized knowledge model facilitates integration between different systems and applications.

DFX (Design for X) – Under the "design for X" label, a broad set of specific design guidelines is contained. Each design guideline refers to a given issue that affects product characteristics. The variable X can encompass many different concepts, e.g., Design for Manufacturability ensures the production of individual parts or components that are based on integral design in terms of mechanical engineering.

DTC/TCM (Design to Cost/Target Cost Management) – Cost management techniques describing a systematic approach to controlling product development and manufacturing costs.

LCC/TCO (Life Cycle Cost/Total Cost of Ownership) – Total Cost of Ownership (TCO) is a financial estimate intended to help buyers and owners determine the direct and indirect costs of a product or service. Life Cycle Cost (LCC) is the total cost of ownership throughout the entire life period of an asset.

LCA&E (Life Cycle Assessment/Analysis & Engineering) – Assessment methodology (Life-Cycle Assessment/Analysis) or engineering (Life-Cycle Engineering) of environmental impact associated with all stages of a product's life cycle.

VA&E (Value Analysis & Engineering) – Systematic analysis of functions of various components and materials to reduce costs of goods, products, and services with acceptable loss of performance or functionality.

QFD (Quality Function Deployment) – Method of transforming qualitative user requirements into quantitative parameters, implementing quality-creating functions, and implementing methods for achieving design quality in subsystems and components, and ultimately in specific elements of the manufacturing process.

FMEA/FMECA (Failure Mode and Effects Analysis/Failure Mode, Effects and Criticality Analysis) – Process of reviewing as many components, assemblies, and subsystems as possible to identify potential failure modes in the product and their causes and effects.

TRIZ (Theory of Inventive Problem Solving) – Theory combining systematic problem-solving methodology with analysis and forecasting techniques derived from studying patterns of inventiveness in global patent literature.

DMU (Digital Mock-Up) – Preparation of digital models, sometimes called digital prototyping. Digital models can resemble the final product, allowing for early corrections rather than changes in later phases of the process. Models can be interactive, showing how the user will interact with the product.

CAD/CAE/CAS/CAM (Computer-Aided Design/Engineering/Styling/Manufacturing) – Computer-Aided Design (CAD) is the use of computers to aid in the creation, modification, analysis, or optimization of a design. CAD is one component of the entire digital product development and as such is used together with other tools that are either integrated modules or standalone products, such as Computer-Aided Engineering (CAE), Computer-Aided Manufacturing (CAM), or Computer-Aided Styling (CAS).

FEA/FEM (Finite Element Analysis/Finite Element Method) – The finite element method is a popular numerical method for solving differential equations arising in engineering and mathematical modeling. Typical areas of interest include traditional fields of structural analysis, heat transfer, fluid flow, mass transport, and electromagnetic potential.

CFD (Computational Fluid Dynamics) – A branch of fluid mechanics that uses numerical analysis and data structures to analyze and solve problems involving fluid flows.

CAPP (Computer-Aided Process Planning) – Computer-aided process planning is the use of computer technology to aid the process planning of a part or product in manufacturing. CAPP is the link between CAD and CAM as it provides for the planning of processes to be used to manufacture the designed part.

DES (Discrete Event Simulation) – Modeling the operation of a system, product, or service as a (discrete) sequence of events in time. Each event occurs at a particular instant in time and marks a change of state in the system. A common exercise in learning to build discrete event simulations is modeling a queuing system, such as customers arriving at a store.

VR/AR (Virtual Reality/Augmented Reality) – Virtual Reality (VR) is a simulated experience that employs pose tracking and 3D near-eye displays to give the user an immersive feel of a virtual world. Augmented Reality (AR) is an interactive experience that combines the real world and computer-generated 3D content.

DMS (Document Management System) – A document management system (DMS) is a computer system used to store, share, track and manage files or documents. Some systems include history tracking where different versions created and modified by different users are recorded.

WMS (Workflow Management System) – A workflow management system (WMS) provides an infrastructure for the set-up, performance and monitoring of a defined sequence of tasks, arranged as a workflow application. For example, in a manufacturing environment, a design document might be automatically routed from a designer to the engineering manager and then to a manufacturing engineer.

ERP/SCM/CRM/SRM (Enterprise Resource Planning/Supply Chain Management/Customer Relationship Management/Supplier Relationship Management) – Enterprise Resource Planning (ERP) is the integrated management of main business processes, often in real-time and mediated by software and technology. ERP is usually referred to as a category of business management software that an organization can use to collect, store, manage, and interpret data from many business activities. Often, Supply Chain Management (SCM), Customer Relationship Management (CRM), or Supplier Relationship Management (SRM) systems are integrated within the ERP system framework.

CMMS (Computerized Maintenance Management System) – A computerized maintenance management system is any software package that maintains a computer database of information on an organization's maintenance operations.

| AREA – ROLES AND COLLABORATION (7 QUESTIONS) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| QUESTIONS | ANSWERS |  |  |  |  |
|  | A | B | C | D | E |
| How is the cross-functional team involved in the product development process? | The cross-functional team does not exist in the product development process. | The cross-functional team exists but is not regularly involved in the process. | The cross-functional team is involved in most stages of product development. | The cross-functional team is involved in the entire product development process, but collaboration is not yet optimal. | The cross-functional team is fully integrated and actively engaged at all stages of product development. |
|  |  | OrgC: X | OrgB: X | OrgA: X, OrgD: X |  |
| How are all stakeholders engaged in the project team, even when they are globally dispersed? | Stakeholders are not engaged in the project team. | Some stakeholders are engaged, but only locally. | Most stakeholders are engaged, but not always, especially when they are globally dispersed. | Stakeholders are engaged even when globally dispersed, but collaboration still requires improvement. | All stakeholders are fully engaged in the project team, regardless of geographic location. |
|  |  | OrgC: X | OrgA: X, OrgD: X | OrgB: X |  |
| How clearly defined are the roles and responsibilities of each individual? | Roles and responsibilities are not clearly defined. | Roles and responsibilities are defined, but often not clear or understood by everyone. | Most roles and responsibilities are clearly defined, but ambiguities still occur. | Roles and responsibilities are well-defined for most team members. | Roles and responsibilities are fully clear, well understood, and respected by all team members. |
|  |  | OrgA: X, OrgD: X | OrgC: X | OrgB: X |  |
| How is high flexibility in task execution ensured in the project? | There is no flexibility in task execution; everything is strictly defined. | Flexibility exists but is limited and applies only to certain tasks. | Flexibility in task execution is possible, but not for all team members. | Flexibility in task execution is ensured, but not always maximally utilized. | Flexibility in task execution is high and supports team efficiency at all stages of the project. |
|  |  | OrgB: X, OrgD: X | OrgA: X | OrgC: X |  |
| What is the technical background and engagement level of the product managers in the projects? | Product managers are unlikely to have a technical background. | Some product managers have a technical background, but it is not the standard. | There are product managers with a medium level of technical background, or only some product managers have it. | Most product managers have an appropriate technical background and are engaged in the projects. | All product managers have a technical background, confirmed by experience, and are fully engaged in development. |
|  |  | OrgD: X | OrgB: X, OrgC: X | OrgA: X |  |
| How fully is the customer engaged in the product development process? | The customer is not engaged in the product development process. | The customer is engaged only in selected projects and rarely fully. | The customer is sometimes engaged in product development but not at every stage. | The customer is regularly engaged, but collaboration is not optimal at all stages. | The customer is fully engaged at all stages of product development, regardless of the product. |
|  |  | OrgB: X, OrgD: X | OrgA: X, OrgC: X |  |  |
| Are experienced designers engaged from the earliest stages of the project? | Experienced designers are not engaged in the early stages of projects. | Experienced designers are engaged, but only sporadically in the early stages. | Experienced designers are engaged in the early stages of most projects. | Experienced designers are engaged in the early stages of all projects, but their involvement is not always optimal. | Experienced designers are always engaged from the earliest stages of projects, and their input is crucial to the project's success. |
|  |  | OrgC: X, OrgD: X |  | OrgA: X, OrgB: X |  |
| AREA – TRAINING (3 QUESTIONS) |  |  |  |  |  |
| QUESTIONS | ANSWERS |  |  |  |  |
|  | A | B | C | D | E |
| How formal programs support the development of interdisciplinary skills? | There are no formal training programs. | Programs exist, but they are limited and sporadic. | Programs are implemented at an intermediate level but do not cover all employees. | Programs are widely available and regularly applied. | Programs are systematic, well-planned, and include all employees. |
|  |  | OrgA: X, OrgC: X, OrgD: X |  |  | OrgB: X |
| How are individual tutoring sessions offered to support skill development? | Individual tutoring sessions are not available. | Tutoring sessions are available in a limited scope and for selected employees. | Tutoring sessions are available to most employees, but they are rarely individualized. | Tutoring sessions are well-developed, but not always available to all employees. | Tutoring sessions are standardized, individualized, and accessible to all employees. |
|  |  | OrgA: X, OrgC: X, OrgD: X |  |  | OrgB: X |
| How are KPIs used to evaluate training outcomes? | KPIs are not used to evaluate training outcomes. | KPIs are sometimes used, but they are not regularly monitored. | KPIs are used, but not all training outcomes are thoroughly evaluated. | KPIs are systematically applied and cover most training programs. | KPIs are comprehensively used to assess all aspects of training and are regularly updated. |
|  |  | OrgA: X, OrgC: X, OrgD: X |  | OrgB: X |  |
| AREA – ACTIVITIES AND FLOW (8 QUESTIONS) |  |  |  |  |  |
| QUESTIONS | ANSWERS |  |  |  |  |
|  | A | B | C | D | E |
| To what extent is a formal product development model followed and documented? | There is no formal product development model. | The model exists but is rarely followed and documented. | The model is followed, but not always documented. | The model is followed and documented by most teams. | The model is a standard, followed and documented in every project. |
|  | OrgD: X | OrgB: X | OrgC: X |  | OrgA: X |
| To what extent is collaboration a part of the product development process? | Collaboration is minimal and sporadic. | Collaboration exists but is limited to a few key stages. | Collaboration covers most stages but is not systematic. | Collaboration covers nearly all stages of product development. | Collaboration is an integral part of the process, covering all stages. |
|  |  |  | OrgD: X | OrgC: X | OrgA: X, OrgB: X |
| How effectively is a comprehensive set of KPIs used to measure product development performance? | There is no comprehensive set of KPIs. | A set of KPIs exists but is used sporadically. | A comprehensive set of KPIs is used, but not in all phases. | A comprehensive set of KPIs is systematically used in most projects. | A comprehensive set of KPIs is used in all projects and phases of product development. |
|  |  | OrgB: X, OrgC: X, OrgD: X | OrgA: X |  |  |
| At what level is frontloading implemented in the product development process? | Frontloading is not applied. | Frontloading is applied only in some projects. | Frontloading is applied in most projects, but not always effectively. | Frontloading is effectively implemented in all projects. | Frontloading is implemented at a very high level in all projects. |
|  | OrgD: X | OrgA: X, OrgB: X, OrgC: X |  |  |  |
| To what extent are continuous improvement initiatives undertaken? | Continuous improvement initiatives are not undertaken. | Continuous improvement initiatives are undertaken, but rarely. | Continuous improvement initiatives are undertaken, but their impact is limited. | Continuous improvement initiatives are regularly undertaken and have a real impact on the process. | Continuous improvement initiatives are standard and greatly impact process efficiency. |
|  |  | OrgC: X | OrgA: X, OrgD: X | OrgB: X |  |
| How effectively are various solutions designed and poorer solutions eliminated as new information arises? | Various solutions are not designed; the process is rigid. | Various solutions are designed, but the elimination of weaker options is unsystematic. | Various solutions are designed, but eliminating poorer options is not systematic. | Various solutions are designed, and poorer solutions are regularly eliminated. | Various solutions are designed in each project, and weaker options are regularly and effectively eliminated. |
|  |  | OrgB: X, OrgC: X | OrgA: X, OrgD: X |  |  |
| To what extent does the product development process focus on customer value? | The product development process does not focus on customer value. | The process partially focuses on customer value, but it is not a priority. | The process focuses on customer value, but not always. | The process consistently focuses on customer value. | The process fully focuses on maximizing customer value. |
|  |  |  | OrgA: X, OrgD: X | OrgB: X, OrgC: X |  |
| How is the competitive analysis process (reverse engineering) implemented? | The competitive analysis process is not implemented. | The competitive analysis process is implemented but to a limited extent. | The competitive analysis process is systematically implemented, but not fully. | The competitive analysis process is effectively implemented in all projects. | The competitive analysis process is a key part of product development and implemented in every project. |
|  |  | OrgA: X, OrgC: X, OrgD: X |  | OrgB: X |  |
| AREA – DECISION-MAKING (22 QUESTIONS) |  |  |  |  |  |
| QUESTIONS | ANSWERS |  |  |  |  |
|  | A | B | C | D | E |
| How is the process of designing and industrializing parts and modules considered? | The design and industrialization process is not considered. | The design and industrialization process is occasionally considered. | The design and industrialization process is regularly considered, but not always systematically. | The design and industrialization process is considered in all projects and is well-organized. | The design and industrialization process is fully considered and integrated at every stage of product development. |
|  |  |  | OrgA: X, OrgB: X, OrgC: X |  | OrgD: X |
| To what extent is the component and semi-finished product manufacturing process considered? | The manufacturing process for components and semi-finished products is not considered. | The manufacturing process for components and semi-finished products is occasionally considered. | The manufacturing process for components and semi-finished products is considered in most projects. | The manufacturing process for components and semi-finished products is a key element in every project. | The manufacturing process for components and semi-finished products is a priority and considered in every project. |
|  |  | OrgA: X, OrgC: X | OrgB: X |  | OrgD: X |
| At what level is the assembly process of components taken into account in product development? | The component assembly process is not considered. | The component assembly process is occasionally considered. | The component assembly process is taken into account, but not always optimized. | The component assembly process is optimized and considered at all stages. | The component assembly process is a key element of product development and is optimized in all projects. |
|  |  | OrgB: X | OrgA: X | OrgC: X | OrgD: X |
| How important is the testing and experimentation process in product development? | The testing and experimentation process is not considered. | The testing and experimentation process is occasionally considered. | The testing and experimentation process is considered in most cases, but its significance is limited. | The testing and experimentation process is systematically applied and has a major impact on product development. | The testing and experimentation process is of utmost importance and influences every decision. |
|  |  |  |  | OrgA: X, OrgB: X, OrgC: X | OrgD: X |
| At what level is the packaging and storage process considered? | The packaging and storage process is not considered. | The packaging and storage process is occasionally considered. | The packaging and storage process is taken into account, but not in all projects. | The packaging and storage process is considered and optimized in most projects. | The packaging and storage process is crucial and fully considered in every project. |
|  |  |  | OrgA: X, OrgB: X, OrgD: X | OrgC: X |  |
| How is the delivery and distribution process considered? | The delivery and distribution process is not considered. | The delivery and distribution process is occasionally considered. | The delivery and distribution process is considered, but its role is not always critical. | The delivery and distribution process is effectively considered in every project. | The delivery and distribution process is an integral part of every decision-making process. |
|  |  | OrgA: X | OrgC: X, OrgD: X | OrgB: X |  |
| To what extent is customer usage considered in product development? | Customer usage is not considered in the product development process. | Customer usage is occasionally considered. | Customer usage is taken into account, but not always monitored. | Customer usage is regularly monitored and considered at all stages. | Customer usage is continuously monitored and fully considered in design decisions. |
|  |  |  | OrgA: X, OrgC: X | OrgD: X | OrgB: X |
| How is the maintenance and after-sales service process considered? | The maintenance and after-sales service process is not considered. | The maintenance and after-sales service process is occasionally considered. | The maintenance and after-sales service process is considered, but not comprehensively. | The maintenance and after-sales service process is crucial and always considered. | The maintenance and after-sales service process is a priority and considered at all stages of development. |
|  |  | OrgA: X | OrgC: X, OrgD: X | OrgB: X |  |
| At what level is the dismantling, recycling, and disposal process considered? | The dismantling, recycling, and disposal process is not considered. | The dismantling, recycling, and disposal process is occasionally considered. | The dismantling, recycling, and disposal process is taken into account, but its role is limited. | The dismantling, recycling, and disposal process is fully considered in every project. | The dismantling, recycling, and disposal process is integrated into every project and regularly monitored. |
|  | OrgD: X | OrgA: X, OrgB: X |  | OrgC: X |  |
| To what extent is the process of inspection, reuse, updating, and refurbishment considered? | The process of inspection, reuse, updating, and refurbishment is not considered. | The process of inspection, reuse, updating, and refurbishment is occasionally considered. | The process of inspection, reuse, updating, and refurbishment is considered, but not always prioritized. | The process of inspection, reuse, updating, and refurbishment is considered in most projects. | The process of inspection, reuse, updating, and refurbishment is fully considered and integrated into product development. |
|  | OrgD: X | OrgA: X, OrgC: X | OrgB: X |  |  |
| How important is the final product cost/price in the decision-making process? | The final product cost is not considered. | The final product cost is considered, but to a limited extent. | The final product cost is considered, but not always comprehensively. | The final product cost is a key element in decisions in all projects. | The final product cost/price is a priority in every design decision. |
|  |  |  | OrgA: X, OrgD: X | OrgB: X, OrgC: X |  |
| To what extent are development costs/ROI considered in the decision-making process? | Development costs/ROI are not considered. | Development costs/ROI are occasionally considered. | Development costs/ROI are regularly monitored, but not always considered in decisions. | Development costs/ROI are carefully monitored and considered in every project. | Development costs/ROI are a key element in every decision and are carefully monitored. |
|  |  |  | OrgA: X, OrgB: X | OrgC: X, OrgD: X |  |
| How important are lifecycle and operational costs, such as maintenance, services, recycling, and disposal? | Lifecycle and operational costs are not considered. | Lifecycle and operational costs are occasionally considered. | Lifecycle and operational costs are considered, but not comprehensively. | Lifecycle and operational costs are systematically considered and greatly influence decisions. | Lifecycle and operational costs are integral to the decision-making process and have a major influence on decisions. |
|  | OrgD: X | OrgA: X, OrgB: X | OrgC: X |  |  |
| How are time-to-market (TTM) and delivery deadlines considered in decision-making? | TTM and delivery deadlines are not considered in decision-making. | TTM and delivery deadlines are considered, but only in some projects. | TTM and delivery deadlines are taken into account, but not always critically. | TTM and delivery deadlines are key factors in every project decision. | TTM and delivery deadlines are the most important criteria in every project decision. |
|  |  |  | OrgA: X, OrgC: X |  | OrgB: X, OrgD: X |
| How does the number of different products impact design decisions? | The number of different products is not considered. | The number of different products is considered to a limited extent. | The number of different products is considered, but not always impactful on final decisions. | The number of different products is considered in most projects and has a direct impact on decisions. | The number of different products is a key factor in every decision and influences design choices. |
|  |  | OrgC: X | OrgA: X | OrgB: X, OrgD: X |  |
| To what extent is product customization considered? | Product customization is not considered. | Product customization is occasionally considered. | Product customization is regularly considered, but not always comprehensively. | Product customization is monitored and considered in every project. | Product customization is fully considered in every project and is a priority in decision-making. |
|  |  | OrgA: X, OrgC: X | OrgD: X | OrgB: X |  |
| How is the level of innovation considered in decision-making? | The level of innovation is not considered. | The level of innovation is considered, but not systematically. | The level of innovation is monitored, but not always comprehensively. | The level of innovation is systematically monitored and considered in all projects. | The level of innovation is monitored and has the greatest impact on every decision. |
|  | OrgD: X |  | OrgA: X, OrgC: X |  | OrgB: X |
| How does brand image influence decision-making? | Brand image is not considered. | Brand image is considered to a limited extent. | Brand image is considered, but not in all decisions. | Brand image is a key factor in design decisions. | Brand image is a priority in every design decision and is closely monitored. |
|  |  |  | OrgA: X, OrgC: X | OrgB: X, OrgD: X |  |
| How important is functional performance (e.g., productivity, speed, accuracy, maintainability)? | Functional performance is not considered. | Functional performance is occasionally considered. | Functional performance is considered, but not always comprehensively. | Functional performance is regularly monitored and considered in every decision. | Functional performance is a priority and considered in all design decisions. |
|  |  |  | OrgA: X, OrgC: X, OrgD: X | OrgB: X |  |
| How is quality performance (e.g., durability, reliability) considered in design decisions? | Quality performance is not considered. | Quality performance is occasionally considered. | Quality performance is considered, but not always a priority. | Quality performance is essential and considered in all decisions. | Quality performance is a priority and the most important element of every design decision. |
|  |  | OrgD: X | OrgA: X, OrgC: X | OrgB: X |  |
| How do compliance and regulatory standards influence decisions? | Compliance with regulations is not considered. | Compliance with regulations is occasionally considered. | Compliance with regulations is considered, but not always comprehensively. | Compliance with regulations is regularly monitored and considered in every decision. | Compliance with regulations and standards is integral to every decision and is closely monitored. |
|  |  |  | OrgB: X, OrgD: X | OrgA: X, OrgC: X |  |
| What other factors, such as environmental sustainability, aesthetics, or social aspects, are considered? | Other factors, such as sustainability, are not considered. | Other factors, such as sustainability, are sometimes considered, but not systematically. | Other factors, such as sustainability, are considered, but not always prioritized. | Other factors, such as sustainability, are regularly considered and significantly influence decisions. | Other factors, such as sustainability, are crucial and prioritized in the decision-making process. |
|  | OrgD: X |  | OrgA: X | OrgB: X, OrgC: X |  |
| AREA – KM PROCESSES (22 QUESTIONS) |  |  |  |  |  |
| QUESTIONS | ANSWERS |  |  |  |  |
|  | A | B | C | D | E |
| How is previous knowledge retrieved during the conceptual phase of a project? | Knowledge from previous projects is not used during the conceptual phase. | Knowledge from previous projects is occasionally used during the conceptual phase. | Knowledge from previous projects is regularly used during the conceptual phase, but not fully systematically. | Knowledge from previous projects is systematically used during the conceptual phase in all projects. | Knowledge from previous projects is fully integrated into every conceptual phase and is prioritized. |
|  |  | OrgA: X, OrgC: X | OrgD: X | OrgB: X |  |
| How is previous knowledge retrieved during the feasibility study? | Knowledge from previous projects is rarely used during the feasibility study. | Knowledge from previous projects is sporadically used during the feasibility study, but not systematically. | Knowledge from previous projects is regularly used during the feasibility study, but not always in every case. | Knowledge from previous projects is considered during the feasibility study in every project. | Knowledge from previous projects is used in every project and has a crucial impact on the feasibility study. |
|  |  | OrgC: X, OrgD: X | OrgA: X | OrgB: X |  |
| How is previous knowledge retrieved during the detailed design phase of the product and its components? | Knowledge from previous projects is not systematically retrieved during the detailed design phase. | Knowledge from previous projects is used, but unsystematically, during the detailed design phase. | Knowledge from previous projects is systematically retrieved, but not always comprehensively. | Knowledge from previous projects is systematically retrieved and used during the detailed design phase. | Knowledge from previous projects is fully retrieved and used during the detailed design phase. |
|  |  | OrgA: X, OrgC: X | OrgD: X | OrgB: X |  |
| How is previous knowledge retrieved during testing, prototyping, and experimentation? | Knowledge from previous projects is not retrieved during testing, prototyping, and experimentation. | Knowledge from previous projects is sometimes retrieved during testing and prototyping. | Knowledge from previous projects is regularly retrieved during testing, prototyping, and experimentation, but not always. | Knowledge from previous projects is fully retrieved during testing and prototyping. | Knowledge from previous projects is an integral part of testing, prototyping, and experimentation. |
|  |  | OrgA: X, OrgC: X | OrgD: X | OrgB: X |  |
| How is previous knowledge retrieved during the final project verification? | Knowledge from previous projects is not retrieved during the final project verification. | Knowledge from previous projects is sporadically retrieved in the final project verification. | Knowledge from previous projects is retrieved during the final verification, but not comprehensively. | Knowledge from previous projects is retrieved during the final project verification in most cases. | Knowledge from previous projects is a priority in the final project verification process. |
|  |  | OrgA: X, OrgC: X | OrgB: X | OrgD: X |  |
| How does a formal knowledge management plan support the product development process? | A formal knowledge management plan does not exist or is not used. | A formal knowledge management plan exists but is not widely used. | A formal knowledge management plan exists and is implemented, but not in all cases. | A formal knowledge management plan is fully implemented and supports product development in all projects. | A formal knowledge management plan is fully implemented and systematically used in all projects. |
|  | OrgB: X, OrgC: X, OrgD: X | OrgA: X |  |  |  |
| To what extent does the company utilize knowledge resources from previous projects? | The company rarely utilizes knowledge resources from previous projects. | The company sporadically utilizes knowledge resources from previous projects. | The company utilizes knowledge resources, but not all projects are included in a systematic KM process. | The company regularly utilizes knowledge resources from previous projects and applies them to new projects. | The company fully utilizes knowledge resources from previous projects and regularly updates the KM process. |
|  |  | OrgA: X | OrgC: X, OrgD: X | OrgB: X |  |
| How are design principles defined by the company considered (frequently reviewed and updated) in the knowledge management process? | The design principles defined by the company do not support the knowledge management process. | The design principles defined by the company support the KM process, but not always. | The design principles defined by the company support the KM process but are not always fully updated. | The design principles defined by the company are regularly updated and support the KM process. | The design principles defined by the company are continuously updated and fully support the KM process. |
|  |  | OrgA: X, OrgB: X, OrgC: X, OrgD: X |  |  |  |
| How are design principles defined by external entities (e.g., clients, suppliers) considered (frequently reviewed and updated) in the KM process? | External design principles are not considered in the KM process. | External design principles are considered, but to a limited extent. | External design principles are considered, but not in all projects. | External design principles are regularly considered and have a significant impact on projects. | External design principles are crucial and are systematically considered in all projects. |
|  |  | OrgB: X, OrgC: X | OrgA: X | OrgD: X |  |
| How are design principles defined by written design rules textbooks/standards considered (frequently reviewed and updated) in the KM process? | Design principles defined in textbooks/standards are not considered in the KM process. | Design principles defined in textbooks/standards are considered, but to a limited extent. | Design principles defined in textbooks/standards are considered, but not in all projects. | Design principles defined in textbooks/standards are regularly considered and have a significant impact on projects. | Design principles defined in textbooks/standards are crucial and are systematically considered in all projects. |
|  | OrgD: X | OrgB: X, OrgC: X | OrgA: X |  |  |
| How do knowledge management principles support the consideration of final product costs? | Knowledge management does not consider final product costs. | Knowledge management considers final product costs, but not always systematically. | Knowledge management considers final product costs, but not comprehensively. | Knowledge management systematically considers final product costs and impacts decisions. | Knowledge management fully considers final product costs and is an integral part of decision-making. |
|  |  | OrgA: X, OrgC: X, OrgD: X | OrgB: X |  |  |
| How does knowledge management influence the assessment of development costs and ROI? | Knowledge management does not influence the assessment of development costs and ROI. | Knowledge management influences the assessment of development costs and ROI, but it is not always a key factor. | Knowledge management influences the assessment of development costs, but not all projects are included in ROI analysis. | Knowledge management has a major influence on the assessment of development costs and ROI in all projects. | Knowledge management has a crucial influence on the assessment of development costs and ROI and is applied in all projects. |
|  |  | OrgA: X, OrgC: X | OrgB: X, OrgD: X |  |  |
| How does knowledge management support the consideration of lifecycle and operational costs (e.g., maintenance, recycling)? | Lifecycle and operational costs are not considered in the KM process. | Lifecycle and operational costs are considered, but to a limited extent. | Lifecycle and operational costs are considered in most projects, but not comprehensively. | Lifecycle and operational costs are fully considered in most projects. | Lifecycle and operational costs are fully considered and have the greatest impact on design decisions. |
|  | OrgD: X | OrgA: X, OrgC: X | OrgB: X |  |  |
| How does knowledge management influence time-to-market (TTM) and delivery deadlines? | Time-to-market and delivery deadlines are not supported by knowledge management. | Time-to-market is considered, but it does not have a critical impact on decisions. | Time-to-market is considered in the KM process but does not always have a major impact. | Time-to-market is regularly considered and influences KM decisions. | Time-to-market is crucial and fully considered in the KM process. |
|  | OrgD: X | OrgA: X, OrgC: X | OrgB: X |  |  |
| How does KM support the evaluation of the number of different products in the company's portfolio? | KM does not support the evaluation of the number of different products in the portfolio. | KM supports the evaluation of the number of different products, but not systematically. | KM supports the evaluation of the number of products, but not systematically in all cases. | KM supports the evaluation of the number of different products and impacts strategic decisions. | KM fully supports the evaluation of the number of different products and is integral to the decision-making process. |
|  |  | OrgA: X, OrgC: X, OrgD: X |  | OrgB: X |  |
| To what extent does KM consider the level of product customization to meet client needs? | The level of product customization is not considered in the KM process. | The level of product customization is considered, but not in all projects. | The level of product customization is regularly considered, but not comprehensively. | The level of product customization is a key factor and is considered in most projects. | The level of product customization is fully considered and has a major impact on design decisions. |
|  |  | OrgA: X, OrgC: X | OrgB: X | OrgD: X |  |
| How does knowledge management support innovation within the company? | The innovation process is not supported by KM. | KM supports innovation, but to a limited extent. | KM supports innovation in most projects, but not always systematically. | KM supports innovation in all projects and has an impact on product development. | KM is a priority in innovation and supports all projects comprehensively. |
|  | OrgD: X | OrgA: X, OrgC: X | OrgB: X |  |  |
| How do KM principles support building brand image? | Brand image is not supported by KM principles. | Brand image is supported by KM, but it is not a priority. | Brand image is considered in most projects, but does not always have a major impact. | Brand image is a key element in KM and influences design decisions. | Brand image is fully integrated with KM and has a major influence on decisions. |
|  |  | OrgA: X, OrgC: X, OrgD: X | OrgB: X |  |  |
| How does knowledge management support the functional performance of products (e.g., productivity, speed, accuracy)? | Functional performance is not monitored as part of KM. | Functional performance is considered, but not regularly monitored. | Functional performance is considered, but not monitored in all projects. | Functional performance is systematically monitored and considered in the KM process. | Functional performance is integral to KM and has a significant impact on decisions. |
|  | OrgD: X | OrgA: X, OrgC: X |  | OrgB: X |  |
| How does knowledge management support monitoring product quality performance (e.g., durability, reliability)? | Quality performance is not considered in the KM process. | Quality performance is considered, but not comprehensively. | Quality performance is considered, but it is not always a priority. | Quality performance is prioritized and regularly monitored in the KM process. | Quality performance is prioritized and fully monitored in all projects. |
|  |  | OrgA: X, OrgC: X | OrgD: X | OrgB: X |  |
| How does knowledge management support compliance with regulations and legal standards? | Knowledge management does not support compliance with regulations and standards. | Knowledge management considers compliance with regulations, but not as a priority. | Knowledge management supports compliance with regulations, but not comprehensively. | Knowledge management supports compliance with regulations and standards in most projects. | Knowledge management fully supports compliance with regulations and standards and has a major influence on decisions. |
|  |  |  | OrgA: X, OrgD: X | OrgB: X, OrgC: X |  |
| What other aspects (e.g., environmental sustainability, aesthetics) are considered in the KM process? | Other aspects, such as environmental sustainability, are not considered in KM. | Other aspects, such as environmental sustainability, are occasionally considered, but not systematically. | Other aspects, such as environmental sustainability, are considered, but not systematically in all projects. | Other aspects, such as environmental sustainability, are regularly considered and influence decisions. | Other aspects, such as environmental sustainability, are integral to the KM process and have a major impact on design decisions. |
|  | OrgD: X | OrgA: X, OrgB: X |  | OrgC: X |  |
| AREA – KM TECHNIQUES (11 QUESTIONS) |  |  |  |  |  |
| QUESTIONS | ANSWERS |  |  |  |  |
|  | A | B | C | D | E |
| To what extent is verbal communication with colleagues used for knowledge sharing? | Verbal communication with colleagues is rarely used for knowledge sharing. | Verbal communication with colleagues is occasionally used for knowledge sharing. | Verbal communication with colleagues is regularly used for knowledge sharing, but not in all cases. | Verbal communication with colleagues is effectively used for knowledge sharing in all projects. | Verbal communication with colleagues is a priority and fully used for knowledge sharing at all project stages. |
|  |  |  | OrgD: X | OrgA: X, OrgB: X, OrgC: X |  |
| How frequently are documents with lessons learned created and utilized? | Documents with lessons learned are not regularly created or used. | Documents with lessons learned are sometimes created, but rarely used. | Documents with lessons learned are regularly created and used in some projects. | Documents with lessons learned are systematically created and used in most projects. | Documents with lessons learned are fully integrated into the process and regularly used in all projects. |
|  |  | OrgA: X, OrgC: X |  | OrgB: X, OrgD: X |  |
| To what extent are design specifications used in knowledge management processes? | Design specifications are rarely used for knowledge sharing. | Design specifications are occasionally used for knowledge sharing, but not systematically. | Design specifications are systematically used for knowledge sharing, but not in all projects. | Design specifications are fully used for knowledge sharing in all projects. | Design specifications are fully and systematically used for knowledge sharing in every project. |
|  |  | OrgA: X, OrgC: X, OrgD: X |  |  | OrgB: X |
| How effectively do questionnaires and checklists support knowledge management? | Questionnaires and checklists are not effectively used in knowledge management. | Questionnaires and checklists are used, but their effectiveness is limited. | Questionnaires and checklists are effectively used, but not prioritized in knowledge management. | Questionnaires and checklists are effectively used and have a major impact on knowledge management. | Questionnaires and checklists are a priority and effectively support knowledge management at every stage. |
|  | OrgD: X | OrgA: X, OrgC: X | OrgB: X |  |  |
| To what extent are meeting rooms, posters, and visual management used for knowledge sharing? | Meeting rooms, posters, and visual management are rarely used for knowledge sharing. | Meeting rooms, posters, and visual management are occasionally used for knowledge sharing. | Meeting rooms, posters, and visual management are regularly used for knowledge sharing, but not in all cases. | Meeting rooms, posters, and visual management are fully used for knowledge sharing. | Meeting rooms, posters, and visual management are a key part of the knowledge-sharing process and are fully used. |
|  | OrgD: X | OrgA: X, OrgC: X |  | OrgB: X |  |
| How often are shared network folders used for storing and sharing knowledge? | Shared network folders are rarely used for storing and sharing knowledge. | Shared network folders are occasionally used for storing and sharing knowledge, but not systematically. | Shared network folders are systematically used for storing and sharing knowledge, but not comprehensively. | Shared network folders are regularly used and support the knowledge management process. | Shared network folders are a priority for storing and sharing knowledge, regularly updated and used. |
|  |  |  |  | OrgA: X, OrgC: X | OrgB: X, OrgD: X |
| To what extent does the intranet support the knowledge management process? | The intranet is not regularly used to support knowledge management. | The intranet is occasionally used to support knowledge management, but not always effectively. | The intranet is regularly used to support knowledge management, but not always efficiently. | The intranet is effectively used to support knowledge management in all projects. | The intranet is an integral tool supporting knowledge management and is used in all projects. |
|  |  | OrgC: X |  | OrgA: X, OrgD: X | OrgB: X |
| How are websites used to work on collaborative projects (e.g., Wiki) in knowledge management? | Websites used to work on collaborative projects are not used for knowledge management. | Websites used to work on collaborative projects are used to a limited extent in knowledge management. | Websites used to work on collaborative projects are used in most cases, but are not yet optimal. | Websites used to work on collaborative projects are effectively used for knowledge management. | Websites used to work on collaborative projects are fully integrated into the knowledge management process and support projects at all stages. |
|  |  | OrgC: X | OrgA: X, OrgD: X | OrgB: X |  |
| How are blogs, forums, and bulletin boards used in knowledge management? | Blogs, forums, and bulletin boards are not used for knowledge management. | Blogs, forums, and bulletin boards are used to a limited extent in knowledge management. | Blogs, forums, and bulletin boards are used in most cases, but are not yet optimal. | Blogs, forums, and bulletin boards are effectively used for knowledge management. | Blogs, forums, and bulletin boards are fully integrated into the knowledge management process and support projects at all stages. |
|  | OrgD: X | OrgC: X |  | OrgA: X | OrgB: X |
| How do PDM/PLM systems support knowledge management? | PDM/PLM systems are not effectively used for knowledge management. | PDM/PLM systems are occasionally used for knowledge management, but not systematically. | PDM/PLM systems are effectively used in most projects, but not always comprehensively. | PDM/PLM systems are fully integrated into the knowledge management process in all projects. | PDM/PLM systems are fully implemented and support knowledge management at every stage of product development. |
|  | OrgD: X | OrgB: X, OrgC: X | OrgA: X |  |  |
| What benefits do KBE software and design automation bring to knowledge management? | KBE software and design automation are not used for knowledge management. | KBE software and design automation are used to a limited extent in knowledge management. | KBE software and design automation are regularly used, but not comprehensively. | KBE software and design automation are effectively used in knowledge management in most projects. | KBE software and design automation are an integral part of knowledge management and are used in all projects. |
|  | OrgD: X | OrgA: X, OrgC: X |  | OrgB: X |  |
| AREA – METHODS (11 QUESTIONS) |  |  |  |  |  |
| QUESTIONS | ANSWERS |  |  |  |  |
|  | A | B | C | D | E |
| To what extent are principles of modularization and standardization of parts used in the design process? | Principles of modularization and standardization of parts are not used in the design process. | Principles of modularization and standardization of parts are applied sporadically. | Principles of modularization and standardization of parts are regularly applied, but not in all projects. | Principles of modularization and standardization of parts are effectively applied in all projects. | Principles of modularization and standardization of parts are a priority and applied in all projects at every stage. |
|  |  |  | OrgA: X, OrgC: X, OrgD: X | OrgB: X |  |
| To what extent is Design for X (DFX) applied in terms of functional performance? | Design for X (DFX) is not applied in terms of functional performance. | Design for X (DFX) is applied, but to a limited extent in terms of functional performance. | Design for X (DFX) is applied in most projects, but not fully comprehensively. | Design for X (DFX) is a key element in projects in terms of functional performance. | Design for X (DFX) is an integral part of every project and fully supports functional performance. |
|  |  | OrgC: X | OrgB: X, OrgD: X | OrgA: X |  |
| How does Design for X (DFX) support product quality performance? | Design for X (DFX) does not support product quality performance. | Design for X (DFX) supports product quality performance, but it is not common. | Design for X (DFX) supports product quality performance in most cases, but not always. | Design for X (DFX) fully supports product quality performance. | Design for X (DFX) is a priority and fully supports product quality performance. |
|  |  | OrgC: X | OrgB: X, OrgD: X | OrgA: X |  |
| To what extent does Design for X (DFX) take into account other customer-perceptible factors, such as aesthetics or eco-design? | Design for X (DFX) rarely takes into account other customer-perceptible factors, such as aesthetics or eco-design. | Design for X (DFX) sometimes considers other factors, like aesthetics or eco-design, but not systematically. | Design for X (DFX) regularly considers other customer-perceptible factors, but not always systematically. | Design for X (DFX) considers factors like aesthetics and eco-design in most projects. | Design for X (DFX) considers all customer-perceptible factors, such as aesthetics and eco-design, and is crucial in every project. |
|  | OrgD: X |  | OrgB: X, OrgC: X | OrgA: X |  |
| How effectively is Design to Cost (DTC) and Target Cost Management (TCM) applied? | Design to Cost (DTC) and Target Cost Management (TCM) are not applied. | Design to Cost (DTC) and Target Cost Management (TCM) are applied, but infrequently. | Design to Cost (DTC) and Target Cost Management (TCM) are applied in most projects, but not always comprehensively. | Design to Cost (DTC) and Target Cost Management (TCM) are effectively applied in all projects. | Design to Cost (DTC) and Target Cost Management (TCM) are integral to every project and have a major impact on costs. |
|  |  | OrgB: X, OrgD: X | OrgA: X, OrgC: X |  |  |
| To what extent is Lifecycle Cost Analysis (LCC) and Total Cost of Ownership (TCO) considered in the design process? | Lifecycle Cost Analysis (LCC) and Total Cost of Ownership (TCO) are not considered in the design process. | Lifecycle Cost Analysis (LCC) and Total Cost of Ownership (TCO) are occasionally considered in the design process. | Lifecycle Cost Analysis (LCC) and Total Cost of Ownership (TCO) are regularly considered, but not comprehensively. | Lifecycle Cost Analysis (LCC) and Total Cost of Ownership (TCO) are fully considered in the design process. | Lifecycle Cost Analysis (LCC) and Total Cost of Ownership (TCO) are fully considered at every stage of the design process. |
|  |  | OrgB: X, OrgD: X |  | OrgC: X | OrgA: X |
| How effectively is Lifecycle Analysis and Engineering (LCA&E) conducted? | Lifecycle Analysis and Engineering (LCA&E) is not conducted. | Lifecycle Analysis and Engineering (LCA&E) is conducted, but to a limited extent. | Lifecycle Analysis and Engineering (LCA&E) is conducted in most projects, but not always. | Lifecycle Analysis and Engineering (LCA&E) is effectively conducted in all projects. | Lifecycle Analysis and Engineering (LCA&E) is a priority in every project and systematically conducted. |
|  |  | OrgB: X, OrgC: X, OrgD: X | OrgA: X |  |  |
| How does Value Analysis and Engineering (VA&E) support the design process? | Value Analysis and Engineering (VA&E) is not used in the design process. | Value Analysis and Engineering (VA&E) is used, but its impact is limited. | Value Analysis and Engineering (VA&E) is used, but its impact is not yet maximized. | Value Analysis and Engineering (VA&E) is used in all projects and has a significant impact on the design process. | Value Analysis and Engineering (VA&E) is a key element of every project and significantly impacts the final outcome. |
|  |  | OrgC: X, OrgD: X |  | OrgA: X, OrgB: X |  |
| How effectively are Quality Functions (QFD) applied in the design process? | Quality Functions (QFD) are not applied in the design process. | Quality Functions (QFD) are applied, but not in all projects. | Quality Functions (QFD) are applied systematically, but not at all project stages. | Quality Functions (QFD) are regularly applied in every project and support the entire development process. | Quality Functions (QFD) are fully applied at every stage of product development and are an integral part of the process. |
|  | OrgD: X |  | OrgB: X | OrgC: X | OrgA: X |
| To what extent does Failure Modes and Effects Analysis (FMEA/FMECA) support product development? | Failure Modes and Effects Analysis (FMEA/FMECA) is not applied. | Failure Modes and Effects Analysis (FMEA/FMECA) is applied, but its effectiveness is limited. | Failure Modes and Effects Analysis (FMEA/FMECA) is effectively applied in most projects. | Failure Modes and Effects Analysis (FMEA/FMECA) is applied systematically and has a major impact on product development. | Failure Modes and Effects Analysis (FMEA/FMECA) is applied in all projects and has a crucial impact on design decisions. |
|  |  |  | OrgC: X | OrgB: X | OrgA: X, OrgD: X |
| To what extent are systematic innovation methods, such as TRIZ, applied in the design process? | Systematic innovation methods, such as TRIZ, are not applied. | Systematic innovation methods, such as TRIZ, are applied to a limited extent. | Systematic innovation methods, such as TRIZ, are regularly applied, but not always comprehensively. | Systematic innovation methods, such as TRIZ, are applied in most projects and support innovation. | Systematic innovation methods, such as TRIZ, are fully integrated into the design process and applied in all projects. |
|  | OrgD: X | OrgB: X, OrgC: X | OrgA: X |  |  |
| AREA – COMPUTERIZATION AND SOFTWARE (22 QUESTIONS) |  |  |  |  |  |
| QUESTIONS | ANSWERS |  |  |  |  |
|  | A | B | C | D | E |
| To what extent does office automation (e.g., spreadsheets, word processors) support the product development process? | Office automation is not used to support the product development process. | Office automation is occasionally used to support the product development process. | Office automation is regularly used to support the product development process, but not in all projects. | Office automation is effectively used to support the product development process in all projects. | Office automation is a priority and fully supports the product development process in every project. |
|  |  |  |  | OrgA: X, OrgB: X, OrgC: X | OrgD: X |
| To what extent are 2D CAD systems used in the design process? | 2D CAD systems are not commonly used in the design process. | 2D CAD systems are used to a limited extent in the design process. | 2D CAD systems are used in most projects, but not always comprehensively. | 2D CAD systems are fully used in all projects. | 2D CAD systems are fully integrated into each project and support the entire design process. |
|  |  | OrgC: X |  | OrgA: X | OrgB: X, OrgD: X |
| What benefits do 3D CAD systems bring to product development? | 3D CAD systems are rarely used in the product development process. | 3D CAD systems are used, but not in all projects. | 3D CAD systems are used in most projects and support product development, but not always fully. | 3D CAD systems are effectively used and support product development in all projects. | 3D CAD systems are a priority and support product development at every stage. |
|  |  | OrgC: X | OrgA: X |  | OrgB: X, OrgD: X |
| To what extent do digital models (DMU) support product development? | Digital models (DMU) are not used in the product development process. | Digital models (DMU) are used, but not systematically. | Digital models (DMU) are regularly used, but not always comprehensively. | Digital models (DMU) are used in all projects and have a major impact on product development. | Digital models (DMU) are an integral part of each project and fully support product development. |
|  |  | OrgA: X, OrgC: X |  |  | OrgB: X, OrgD: X |
| To what extent computer-aided styling (CAS) is used in product development projects? | Computer-aided styling (CAS) is not used in projects. | Computer-aided styling (CAS) is occasionally used in projects. | Computer-aided styling (CAS) is used in most projects, but not always comprehensively. | Computer-aided styling (CAS) is effectively used in all projects. | Computer-aided styling (CAS) is a key element in each project. |
|  | OrgD: X | OrgC: X | OrgA: X, OrgB: X |  |  |
| To what extent does computer-aided engineering (CAE) support the design process? | Computer-aided engineering (CAE) does not support the design process. | Computer-aided engineering (CAE) is used, but not in all projects. | Computer-aided engineering (CAE) is used in most projects, but not always fully. | Computer-aided engineering (CAE) is fully used in all projects. | Computer-aided engineering (CAE) is fully integrated into each project and supports the entire development process. |
|  |  | OrgC: X | OrgA: X |  | OrgB: X, OrgD: X |
| To what extent finite element analysis (FEA/FEM) method is used in design processes? | The finite element analysis (FEA/FEM) method is not used for design. | The finite element analysis (FEA/FEM) method is applied, but not in all cases. | The finite element analysis (FEA/FEM) method is regularly applied, but not comprehensively. | The finite element analysis (FEA/FEM) method is effectively applied in all projects. | The finite element analysis (FEA/FEM) method is a priority and used in every project. |
|  |  | OrgA: X | OrgC: N/A |  | OrgB: X, OrgD: X |
| To what extent is computational fluid dynamics (CFD) used in projects? | Computational fluid dynamics (CFD) is not used in projects. | Computational fluid dynamics (CFD) is used, but not in all projects. | Computational fluid dynamics (CFD) is used in most projects, but not comprehensively. | Computational fluid dynamics (CFD) is fully used in all projects. | Computational fluid dynamics (CFD) is fully used in all projects and supports the development processes. |
|  |  |  | OrgA: N/A, OrgB: X, OrgC: N/A |  | OrgD: X |
| To what extent KBE and design automation support the design processes? | KBE and design automation are not used in the design process. | KBE and design automation are used, but their application is limited. | KBE and design automation are regularly used, but not in all projects. | KBE and design automation are effectively used in most projects. | KBE and design automation are a priority and are applied comprehensively in every project. |
|  | OrgD: X | OrgA: X | OrgC: N/A | OrgB: X |  |
| To what extent is computer-aided manufacturing (CAM) used in the production process? | Computer-aided manufacturing (CAM) is not used in the production process. | Computer-aided manufacturing (CAM) is used occasionally in the production process. | Computer-aided manufacturing (CAM) is used in most projects, but not always comprehensively. | Computer-aided manufacturing (CAM) is systematically used in all projects. | Computer-aided manufacturing (CAM) is fully used in all projects. |
|  |  | OrgC: X | OrgA: X, OrgB: X, OrgD: X |  |  |
| To what extent does computer-aided process planning (CAPP)/digital factories support product development process? | Computer-aided process planning (CAPP)/digital factories does not support product development process. | Computer-aided process planning (CAPP)/digital factories is sporadically used in product development process. | Computer-aided process planning (CAPP)/digital factories is regularly used, but not in all projects. | Computer-aided process planning (CAPP)/digital factories is fully used in the product development process. | Computer-aided process planning (CAPP)/digital factories is an integral part of each product development process. |
|  |  | OrgC: X, OrgD: X | OrgB: X | OrgA: X |  |
| To what extent does discrete event simulation (DES) support product development processes? | Discrete event simulation (DES) does not support product development processes. | Discrete event simulation (DES) is applied, but not comprehensively. | Discrete event simulation (DES) is applied in most projects, but not always systematically. | Discrete event simulation (DES) is applied in all projects and supports product development processes. | Discrete event simulation (DES) is a priority and fully supports all product development processes. |
|  |  | OrgC: X | OrgA: N/A, OrgD: N/A | OrgB: X |  |
| To what extent virtual reality (VR) and augmented reality (AR) are used in product development? | Virtual reality (VR) and augmented reality (AR) are not used in product development. | Virtual reality (VR) and augmented reality (AR) are applied, but not in all projects. | Virtual reality (VR) and augmented reality (AR) are applied in most projects, but not comprehensively. | Virtual reality (VR) and augmented reality (AR) are effectively used in all projects. | Virtual reality (VR) and augmented reality (AR) are fully integrated into each project and have a significant impact on product development. |
|  |  | OrgA: X, OrgB: X | OrgC: N/A, OrgD: N/A |  |  |
| How do PDM/PLM systems support product data management? | PDM/PLM systems do not support product data management. | PDM/PLM systems are applied, but not systematically in all projects. | PDM/PLM systems are regularly applied in most projects, but not always comprehensively. | PDM/PLM systems are fully applied in all projects and support product data management. | PDM/PLM systems are fully applied in all projects and are a key tool in product data management. |
|  |  | OrgB: X, OrgC: X | OrgA: X, OrgD: X |  |  |
| What benefits do document management systems (DMS) bring to projects? | Document management systems (DMS) are not used in projects. | Document management systems (DMS) are used, but not always comprehensively. | Document management systems (DMS) are regularly applied, but not in all projects. | Document management systems (DMS) are fully applied in all projects. | Document management systems (DMS) are an integral part of every project and fully support document management. |
|  |  | OrgC: X, OrgD: X | OrgB: X | OrgA: X |  |
| To what extent does workflow management system (WMS) support project management? | Workflow management system (WMS) does not support project management. | Workflow management system (WMS) is sporadically used in projects. | Workflow management system (WMS) is regularly used, but not comprehensively. | Workflow management system (WMS) is effectively used in all projects. | Workflow management system (WMS) is fully used in each project. |
|  |  | OrgC: X, OrgD: X | OrgA: X, OrgB: X |  |  |
| To what extent is enterprise resource planning (ERP) applied in product development processes? | Enterprise resource planning (ERP) is not applied in product development processes. | Enterprise resource planning (ERP) is applied, but not in all projects. | Enterprise resource planning (ERP) is regularly applied, but not comprehensively. | Enterprise resource planning (ERP) is applied in all projects and supports product development processes. | Enterprise resource planning (ERP) is a key tool supporting product development processes and is fully applied. |
|  |  | OrgB: X, OrgC: X | OrgA: X |  | OrgD: X |
| How does supply chain management (SCM) support product development? | Supply chain management (SCM) does not support product development. | Supply chain management (SCM) supports product development, but not systematically. | Supply chain management (SCM) supports product development, but not comprehensively. | Supply chain management (SCM) supports product development in all projects. | Supply chain management (SCM) is fully integrated in every project and fully supports development processes. |
|  |  | OrgB: X, OrgC: X |  | OrgA: X, OrgD: X |  |
| To what extent do customer relationship management (CRM) systems impact product development? | Customer relationship management (CRM) systems do not impact product development. | Customer relationship management (CRM) systems have limited impact on product development. | Customer relationship management (CRM) systems are regularly applied, but not in all projects. | Customer relationship management (CRM) systems are fully applied in all projects. | Customer relationship management (CRM) systems are critical for product development and fully applied. |
|  | OrgD: X | OrgA: X, OrgC: X | OrgB: X |  |  |
| How do supplier relationship management (SRM) systems support the product development process? | Supplier relationship management (SRM) systems do not support the product development process. | Supplier relationship management (SRM) systems are applied, but not in all cases. | Supplier relationship management (SRM) systems are regularly applied, but not in all projects. | Supplier relationship management (SRM) systems are effectively applied in all projects. | Supplier relationship management (SRM) systems are a priority and are applied comprehensively in each project. |
|  |  | OrgA: X, OrgB: X, OrgC: X | OrgD: X |  |  |
| To what extent is a computerized maintenance management system (CMMS) applied in product development processes? | A computerized maintenance management system (CMMS) is not applied in product development processes. | A computerized maintenance management system (CMMS) is applied to a limited extent. | A computerized maintenance management system (CMMS) is regularly applied, but not comprehensively. | A computerized maintenance management system (CMMS) is applied in all projects. | A computerized maintenance management system (CMMS) is integral to each project and supports all product development processes. |
|  |  | OrgB: X | OrgA: N/A. OrgC: N/A, OrgD: N/A |  |  |
| To what extent does Lifecycle Analysis software (LCA) affect product development? | Lifecycle Analysis software (LCA) does not affect product development. | Lifecycle Analysis software (LCA) has limited impact on product development. | Lifecycle Analysis software (LCA) is regularly used, but not in all projects. | Lifecycle Analysis software (LCA) is fully utilized in all projects. | Lifecycle Analysis software (LCA) is crucial to product development and are used to their full extent. |
|  |  | OrgA: X, OrgB: X, OrgC: X, OrgD: X |  |  |  |
| To what extent do software project management systems affect product development? | Software project management systems do not affect product development. | Software project management systems have limited impact on product development. | Software project management systems are regularly used, but not in all projects. | Software project management systems are fully utilized in all projects. | Software project management systems are crucial to product development and are used to their full extent. |
|  |  | OrgA: X, OrgC: X | OrgB: X, OrgD: X |  |  |